Open Rhino and Grasshopper file in following folder:

ifmm_lib\production\11_IFMM_NEST_FinalWall_Fab

FABRICATION INTERFACE

A 	Opening the files

Once the GH file is opened, you need to do the following steps:

- Press the init button from the fabrication component.

- Press the following 4 buttons once:

refresh, start_timer, stop_timer, self_update (false>true>false)

B 	Setting the starting node

You can define from which node to start the fabrication by setting the parameters of the 
 row number, 
 the index of the node within the row 
 and the surface 1 or surface 2 of the mesh

The preview of this (if not turned off) is the blue tool head:

Since this only a preview, you need to set the node of the fabrication start with another button:  start from task index

Only this sets the node for the fabrication start and sets the preview of the red tool head on top of the blue tool head!

C 	Connecting the clients

Before starting the fabrication, you need to make sure that all clients are connected.

You have two possibilities to do that:

This is the interface to connect to all clients individually. (1. IF robot arm (= ABB Controller), 2. MM tool (= Arduino) 3. Vision (= IF on-board computer))

Once you press connect, you will see the print-out, which tells you which client is off- or online.

Careful --> all client servers must run, before you are able to connect to them.

When you press refresh in the state viewer, the printout box at the top gives you feedback about the connection state of all clients, as well as the state of the fabrication. (Once you start the timer, it will always give you a live update, this uses a lot of processing power though.) For fabricating the mesh, the clients MM tool (Arduino) and IF robot arm need to be online, and the parameter line_est_toggle must be set to False, and correction_toggle must be set to True. For scanning the mesh, the clients Vision and IF robot arm need to be online, and the parameter line_est_toggle must be set to True, and correction_toggle must be set to False.

If you press reset clients, it resets the clients that you choose in the dropdown menu (E.g. “ROBOT”/”ARDUINO”). Usually, this button is used after you stopped the fabrication.

D 	Starting the fabrication or scanning process

Once the  clients are online, the  right parameters are set, and the  correct node index for starting is set, you can press start fabrication.

E 	Stopping the fabrication or scanning process

Usually, the fabrication or scanning process only needs to be stopped only at the end of the day, or when something went wrong. If the Arduino is in the middle of a routine, the button with the two lines (=) on the ABB handheld needs to be pressed, before the fabrication stop button can be pressed.

FYI: 	- 	“Arduino enable button”

= 	“Arduino stop button”

When Arduino is out of a routine, the stop button can be pressed. This also stops the connection to the clients. Therefore, before being able to start the fabrication again, the button “reset clients” needs to be pressed.

If you want to see the printouts of the serial port of the Arduino client, you can open the serial monitor from the Arduino IDE. Careful  when the serial monitor is opened, the Arduino program restarts. Therefore, it is better to open it once before connecting, or the connection has to be reset after opening the serial monitor.